Management IP Request Form – Switch / Router

| Section 1. Requestor Information ( NAME IN BLOCK LETTERS ONLY ) |  |
| --- | --- |
| College or Department |  |
| Engineer Full Name : |  |
| Mobile: | Email : |
| Tel: Extension: | ITSM NO / CHANGE NO : |
| Date : |  |

| Section 2.0 Switch / Router IP Address Request details |  |  |  |  |
| --- | --- | --- | --- | --- |
| Device Name – IP requested | Serial No | Device Model |  | Device Type |
|  |  |  |  | Layer 2 Layer 3 |
| Device Location (Complete Detail with Room no ) |  | Installation :New : Maintenance |  |  |

| Section 2.1 Reason for Switch / Router IP Address Request in detail. |  |
| --- | --- |

| Technical Affair Department Internet use only |  |  |  |
| --- | --- | --- | --- |
| IP Address | Vlan Info | Remarks |  |

I agree with terms and conditions stated at the back of this request form.

| Internal use – Deanship of Information Technology – Manager / Vice Dean Approval |  |  |  |
| --- | --- | --- | --- |
| Approval Reject request info | Name | Signature |  |

## NBU Switch / Router Management Policy

Every Switch / Management must meet the following configuration standards:

The enable password on the router or switch must be kept in a secure encrypted form. The router or switch must have the enable password set to the current production router/switch password from the device’s support organization.

- The following services or features must be disabled:

  - IP directed broadcasts

  - Incoming packets at the router/switch sourced with invalid addresses such as RFC1918 addresses

  - TCP small services

  - UDP small services

  - All source routing and switching

  - All web services running on router

  - Telnet, FTP, and HTTP services

  - Auto-configuration

- The following services should be disabled unless a business justification is provided:

  - Discovery protocols

  - Dynamic trunking

  - Scripting environments, such as the TCL shell

- The following services must be configured:

  - Password-encryption

  - NTP configured to a corporate standard source

- Access control lists must be used to limit the source and type of traffic that can terminate on the device itself.

- Access control lists for transiting the device are to be added as business needs arise.

- The router must be included in the corporate enterprise management system with a designated point of contact.

- Telnet may never be used across any network to manage a router, unless there is a secure tunnel protecting the entire communication path. SSH, version 2 is the preferred management protocol.

- The corporate router configuration standard will define the category of sensitive routing and switching devices, and require additional services or configuration on sensitive devices including:

  - IP access list accounting

  - Device logging

  - Incoming packets at the router sourced with invalid addresses, such as RFC1918 addresses, or those that could be used to spoof network traffic shall be dropped

  - Router console and modem access must be restricted by additional security controls.

I agree with the terms and conditions stated in the request form and assure I have configured the device as per above stated policies before adding it into the NBU Networks.

| Internal use – Deanship of Information Technology – Engineer |  |  |
| --- | --- | --- |
| Device Installation Date |  |  |
| Signature |  |  |